SYSTEM ARCHITECTURE

& TECHNICAL DESIGN DOCUMENT

AdaptiveOrchestrator

A Dynamic Edge–Cloud Orchestration Framework

for Resilient Industrial Anomaly Detection

| Field | Detail |
| --- | --- |
| Document type | System Architecture & Technical Design |
| Project | AdaptiveOrchestrator — M.Sc. Research Project |
| Candidate | Ajith Kumara — M.Sc. in Artificial Intelligence |
| Supervisor | Dr. Thushari Silva |
| Institution | University of Moratuwa |
| Version | 1.0 — Initial Release |
| Date | February 2026 |
| Status | Approved for Implementation |
| Environment | WSL2 Ubuntu 22.04 + Docker Desktop 29.2 on Intel i5-1135G7, 16GB RAM |
| Cloud | AWS Free Tier — S3 + CloudWatch + Lambda (Option B) |

# 1.  Research Context and Problem Statement

Industrial IoT systems today rely on static, design-time partitioning of computation between edge devices and cloud infrastructure. The edge always handles lightweight processing. The cloud always handles analytics. This partition never changes at runtime, regardless of network conditions, device load, or anomaly severity.

A systematic review of nine IEEE-published papers (2022–2025) confirmed that no existing system implements runtime-adaptive orchestration. All reviewed systems fix their edge-cloud boundary at deployment time. Three specific failure modes emerge from this static approach:

| Failure Mode | Trigger | Consequence | Evidence in Literature |
| --- | --- | --- | --- |
| Blindness | WAN connection fails | Anomaly detection stops completely — no local fallback | Hareram et al. 2025 — WAN failure causes detection halt |
| Contention | Multiple sensor streams overload edge | Latency spikes propagate through system — detection delayed | Li et al. 2025 — edge overload documented under concurrent load |
| Waste | Network stable but all data sent to cloud | Unnecessary egress charges with no detection improvement | Implicit in all 9 papers — no cost-aware routing mechanism |

Core Research Insight: Every paper reviewed treats infrastructure state as a design-time input — something fixed before deployment. AdaptiveOrchestrator treats it as a runtime variable — something measured continuously and responded to dynamically. This single distinction is the entire research contribution.

# 2.  System Overview

## 2.1  High-Level Architecture

AdaptiveOrchestrator is a four-layer system. The key innovation sits entirely in Layer 2 — the Local Orchestrator. Layers 1, 3, and 4 use standard, well-established components. Layer 2 is what does not exist anywhere in the current literature.

┌─────────────────────────────────────────────────────────────────────┐ │ LAYER 4 — MLOps Loop │ │ Drift Monitor (Lambda) ──→ Retraining Trigger ──→ MLflow Registry │ └─────────────────────────────────────────────────────────────────────┘ ↑ model updates ┌─────────────────────────────────────────────────────────────────────┐ │ LAYER 3 — Cloud Platform (Docker-local + AWS S3/CloudWatch) │ │ Kafka ──→ Spark Streaming ──→ Delta Lake (S3) ──→ SHAP Explainer │ │ ↓ │ │ CloudWatch Dashboard │ └─────────────────────────────────────────────────────────────────────┘ ↑ events / ↓ model ┌─────────────────────────────────────────────────────────────────────┐ │ LAYER 2 — Local Orchestrator ★ THE INNOVATION ★ │ │ Context Monitor (Latency/CPU) + Policy Executor (Routing Logic) │ │ Alert Router (MQTT/Dashboard) + NATS Buffer (outage storage) │ └─────────────────────────────────────────────────────────────────────┘ ↑ sensor data ┌─────────────────────────────────────────────────────────────────────┐ │ LAYER 1 — Edge Device (Docker container: 1 vCPU / 4GB limit) │ │ Sensor Simulator ──→ Inference Engine (Isolation Forest) │ └─────────────────────────────────────────────────────────────────────┘

## 2.2  Three Operational Modes

The Policy Executor continuously evaluates live context signals and switches between three operational modes. Mode transitions are automatic, logged, and published to CloudWatch. No human intervention is required.

| Mode | Trigger Condition | Edge Action | Cloud Action | Alert Action |
| --- | --- | --- | --- | --- |
| EDGE_ONLY (Autonomous) | Latency > 500ms OR WAN unreachable OR CPU > 80% | Run Isolation Forest locally Buffer events in NATS JetStream | No cloud communication Cloud receives nothing | MQTT broadcast on LAN Dashboard alert Lambda SMS/email if critical |
| HYBRID (Collaborative) | High-severity anomaly detected AND network latency < 500ms | Edge detects anomaly Publishes to Kafka | SHAP explainer invoked Explanation stored in Delta Lake | Standard alert pipeline SHAP result logged to CloudWatch |
| CLOUD_OPTIMISED (Full Analytics) | Network stable < 200ms CPU below threshold No critical anomaly | Sensor data published to Kafka topic | Full Spark processing Delta Lake write to S3 Model retraining eligible | Normal operation No alert unless anomaly |

## 2.3  Policy Decision Logic

The policy executor runs the following decision tree every time a context snapshot is received from the Context Monitor. Decisions execute in under 10 milliseconds because the policy table is cached locally — no network call is required.

Sensor Event Ingested │ ▼ ┌───────────────────┐ │ Latency > 500ms ? │──── YES ──→ EDGE_ONLY mode │ OR WAN down ? │ Buffer event locally └───────────────────┘ Alert router activated │ NO ▼ ┌───────────────────┐ │ CPU > 80% ? │──── YES ──→ EDGE_ONLY mode └───────────────────┘ Shed non-critical load │ NO ▼ ┌───────────────────┐ │ Anomaly Critical? │──── YES ──→ HYBRID mode │ score > 0.85 ? │ Edge detects + Cloud explains └───────────────────┘ │ NO ▼ CLOUD_OPTIMISED mode Full analytics pipeline Delta Lake write to S3 Policy cached at edge — decision latency < 10ms

# 3.  Full Technology Stack

## 3.1  Environment

| Dimension | Specification | Justification |
| --- | --- | --- |
| Host OS | Windows 11 — Intel i5-1135G7, 16GB RAM, 8 logical cores | Candidate's development machine |
| Linux layer | WSL2 Ubuntu 22.04 LTS | Required for tc netem, Docker, Python tooling |
| Containerisation | Docker Desktop 29.2 with WSL2 integration | All components run in isolated containers |
| Edge simulation | Docker container limited to 1 vCPU, 4GB RAM via cgroups | Approximates Jetson Nano (4-core ARM, 4GB) profile |
| Network simulation | tc netem on Docker bridge interface (br-xxxxxxxxxx) | Injects latency/loss without physical hardware |
| Cloud integration | AWS Free Tier — S3, CloudWatch, Lambda (ap-south-1) | Real cloud credentials; zero cost at evaluation scale |
| Python version | Python 3.10.12 in virtualenv | All components use shared venv |

## 3.2  Component Stack — Complete Reference

| Layer | Component | Technology | Docker Image | Port | Purpose |
| --- | --- | --- | --- | --- | --- |
| Simulation | Sensor Simulator | Python 3.10 + pandas + numpy | python:3.10-slim | — | Replay CWRU/SKAB at configurable Hz via NATS |
| Messaging | NATS JetStream | NATS 2.x with JetStream enabled | nats:latest | 4222 | Local message bus — edge events, context, alerts |
| Edge | Inference Engine | scikit-learn IsolationForest | python:3.10-slim | 8000 | Local anomaly detection — no cloud dependency |
| Edge | Context Monitor | psutil + subprocess ping | python:3.10-slim | — | Measures latency, CPU, RAM every 1 second |
| Edge | Policy Executor | Python asyncio | python:3.10-slim | — | Core innovation — runtime mode decision engine |
| Edge | NATS Buffer | NATS JetStream persistent store | nats:latest | — | Event queue during WAN outage — max 5000 events |
| Edge | Alert Router | paho-mqtt + FastAPI | python:3.10-slim | 8080 | Multi-channel alert dispatch during outage |
| Monitoring | MQTT Broker | Eclipse Mosquitto | eclipse-mosquitto | 1883 | Local alert distribution on LAN — no internet |
| Monitoring | Prometheus | Prometheus 2.x | prom/prometheus:latest | 9090 | Scrapes /metrics from all components every 15s |
| Monitoring | Grafana | Grafana 10.x | grafana/grafana:latest | 3000 | Live monitoring dashboard — thesis screenshots |
| Streaming | Zookeeper | Apache Zookeeper | confluentinc/cp-zookeeper:7.5 | 2181 | Kafka cluster coordination |
| Streaming | Kafka Broker | Apache Kafka | confluentinc/cp-kafka:7.5 | 9092 | Event streaming edge → cloud pipeline |
| Processing | Spark Master | Apache Spark 3.5 | bitnami/spark:3.5 | 8081 | Stream processing master node |
| Processing | Spark Worker | Apache Spark 3.5 | bitnami/spark:3.5 | — | 2GB RAM, 2 cores — stream processing worker |
| Storage | Delta Lake | delta-spark 3.x | bitnami/spark:3.5 | — | ACID lakehouse on S3 — partitioned by date+mode |
| Explainer | SHAP Explainer | SHAP + FastAPI | python:3.10-slim | 8002 | TreeExplainer for anomaly feature importance |
| MLOps | MLflow | MLflow 2.x | ghcr.io/mlflow/mlflow | 5000 | Model versioning, experiment tracking, artifacts |
| AWS | S3 Bucket | AWS S3 (ap-south-1) | — (managed service) | — | Delta Lake storage, model artifacts, eval logs |
| AWS | CloudWatch | AWS CloudWatch (ap-south-1) | — (managed service) | — | 5 custom metrics — thesis dashboard screenshots |
| AWS | Lambda: alert | Python 3.11 runtime | — (serverless) | — | Anomaly alert dispatcher → SES email on outage |
| AWS | Lambda: drift | Python 3.11 runtime | — (serverless) | — | Daily drift check → S3 report + retraining trigger |

## 3.3  Kafka Topic Design

All inter-component communication uses NATS locally and Kafka for the edge-to-cloud boundary. Topic names follow a hierarchical naming convention for clarity and routing.

| Topic / Subject | Producer | Consumer | Message Schema | Retention |
| --- | --- | --- | --- | --- |
| sensors.raw | Sensor Simulator | Inference Engine | {timestamp, sensor_id, value, label, dataset} | In-memory — high frequency |
| context.snapshot | Context Monitor | Policy Executor | {latency_ms, cpu_pct, ram_pct, cloud_reachable, timestamp} | In-memory — 1s interval |
| inference.result | Inference Engine | Policy Executor | {score, decision, latency_ms, timestamp} | In-memory — per event |
| orchestrator.mode | Policy Executor | All components | {from_mode, to_mode, trigger, context, timestamp} | Persistent — mode changes only |
| buffer.store | Policy Executor | NATS Buffer | {full inference result + mode + cloud_reachable} | Persistent — until flushed |
| alert.trigger | Policy Executor | Alert Router | {sensor_id, score, severity, mode, timestamp} | Persistent — critical events |
| cloud.ingest (Kafka) | Kafka Producer | Spark Streaming | {full event JSON} | 7 days |
| cloud.explain (Kafka) | Kafka Producer | SHAP Explainer | {anomaly event + feature vector} | 7 days |
| shap.result (Kafka) | SHAP Explainer | Delta Lake Writer | {anomaly_id, shap_values[], feature_names[]} | 7 days |

## 3.4  Delta Lake Schema

Delta Lake is partitioned by date and operational mode to enable efficient evaluation queries. All writes go through Spark Structured Streaming to guarantee ACID semantics.

| Column | Type | Description | Example |
| --- | --- | --- | --- |
| event_id | STRING | UUID assigned at ingestion | evt-7f3a-2025-09-15 |
| timestamp | TIMESTAMP | UTC event time from edge | 2025-09-15T10:42:03.441Z |
| sensor_id | STRING | Source sensor identifier | bearing-01 |
| value | DOUBLE | Raw sensor reading | 0.7341 |
| anomaly_score | DOUBLE | Normalised IF score 0.0–1.0 | 0.8731 |
| decision | STRING | ANOMALY or NORMAL | ANOMALY |
| ground_truth | INTEGER | Dataset label: 0=normal, 1=anomaly | 1 |
| mode | STRING | Operational mode at time of event | EDGE_ONLY |
| cloud_reachable | BOOLEAN | Was cloud available at event time | false |
| inference_ms | DOUBLE | Edge inference latency in milliseconds | 4.2 |
| latency_ms | DOUBLE | Network latency at event time | 934.1 |
| cpu_pct | DOUBLE | Edge CPU utilisation at event time | 67.3 |
| shap_values | ARRAY<DOUBLE> | SHAP feature importance (Hybrid mode only) | [0.23, 0.61, 0.08, 0.08] |
| dataset | STRING | Source dataset name | cwru |
| partition_date | DATE | Partition key — date of event | 2025-09-15 |

# 4.  Component Specifications

## 4.1  Sensor Simulator

| Property | Specification |
| --- | --- |
| Purpose | Replays CWRU Bearing and SKAB datasets as live sensor streams over NATS |
| Input datasets | CWRU Bearing Data (vibration, 12kHz) + SKAB (pressure/temp/flow, 50Hz) |
| Synthetic fallback | Generates statistically representative synthetic data if real datasets not present |
| Replay rate | Configurable Hz — default 100Hz for CWRU, 50Hz for SKAB |
| Noise injection | Configurable Gaussian noise level — used for Scenario 3 (satellite link) testing |
| NATS subject | sensors.raw — publishes JSON event per sample |
| Anomaly ratio | ~10% anomaly rate in CWRU synthetic; ~8% in SKAB synthetic |
| Multi-sensor | Simulates up to 10 virtual sensors by replaying SKAB with different seeds |
| Unit test | test_sensor_simulator.py — shape, anomaly ratio, noise injection, NATS delivery |
| Pass criteria | F1 > 0.85 on synthetic data before tuning; zero dropped events at 100Hz for 60s |

## 4.2  Inference Engine

| Property | Specification |
| --- | --- |
| Algorithm | Isolation Forest (scikit-learn) — unsupervised, trained on normal data only |
| Hyperparameters | n_estimators=100, contamination=0.05, random_state=42, n_jobs=-1 |
| Score normalisation | Raw decision_function score mapped to 0–1 via sigmoid: 1/(1+e^(score×5)) |
| Anomaly threshold | score > 0.60 = ANOMALY (validated by threshold sweep in Week 2) |
| Inference latency | Target P99 < 50ms on constrained container (1 vCPU, 4GB RAM) |
| Model persistence | Saved as pickle to /data/models/isolation_forest.pkl + scaler.pkl |
| MLflow tracking | Every training run logged: F1, precision, recall, n_samples, hyperparameters |
| NATS subject | Consumes: sensors.raw — Publishes: inference.result |
| Unit test | test_inference_engine.py — score range, threshold, latency, train/predict cycle |
| Pass criteria | F1 > 0.85 on held-out CWRU; P99 latency < 50ms in constrained container |

## 4.3  Context Monitor

| Property | Specification |
| --- | --- |
| Sampling interval | Every 1 second — continuous loop |
| Latency measurement | subprocess ping to cloud_node (3 samples) — parses min/avg/max/mdev |
| Unreachable detection | ping returncode != 0 OR timeout > 2s → cloud_reachable = false |
| CPU measurement | psutil.cpu_percent(interval=0.1) — system-wide across all cores |
| RAM measurement | psutil.virtual_memory().percent |
| NATS subject | Publishes: context.snapshot every 1 second |
| tc netem coupling | Latency injected on Docker bridge is detected within 1–2 seconds — verified in Sprint 2 |
| Unit test | test_context_monitor.py — latency parse, CPU measure, unreachable handling |
| Pass criteria | Measured latency within ±50ms of tc netem injected value |

## 4.4  Policy Executor — Core Innovation

Why This Component Is the Research Contribution - Every paper in the literature review has a static equivalent of this component — a fixed rule baked in at design time. - This component is the first to make the edge-cloud routing decision at runtime based on live context signals. - The mode history it produces is the primary evidence for the thesis evaluation. - All other components (Kafka, Spark, Delta Lake, SHAP) are standard components. This one is not.

| Property | Specification |
| --- | --- |
| Input | context.snapshot (from Context Monitor) + inference.result (from Inference Engine) |
| Output | Routes events to: buffer.store OR cloud.ingest OR cloud.explain based on mode |
| Mode evaluation | Runs on every context snapshot — O(1) rule evaluation — no network call needed |
| Decision speed | < 10ms — policy table cached locally in Python dict |
| Priority order | 1. Network failure/high latency → EDGE_ONLY 2. CPU overload → EDGE_ONLY 3. Critical anomaly → HYBRID 4. Default → CLOUD_OPTIMISED |
| Transition logging | Every mode change logged to structured JSON with context snapshot and trigger reason |
| CloudWatch publish | Mode value published to CloudWatch every 60 seconds via boto3 |
| NATS subjects | Subscribes: context.snapshot, inference.result — Publishes: orchestrator.mode, buffer.store, alert.trigger, cloud.ingest, cloud.explain |
| Unit test | test_policy_executor.py — all 3 mode decisions, routing logic, transition events |
| Integration test | tc netem 900ms injection → EDGE_ONLY within 5s — verified by automated test |
| Pass criteria | Zero missed mode transitions; correct routing to all 3 destinations verified |

## 4.5  NATS Buffer

| Property | Specification |
| --- | --- |
| Technology | NATS JetStream persistent stream — survives container restarts |
| Max capacity | 5,000 events — sized for 50 seconds at 100Hz during outage |
| Overflow policy | Drop NORMAL events when buffer > 80% full. Never drop ANOMALY events. |
| Critical reserve | Bottom 500 events always reserved for ANOMALY events regardless of fill level |
| Flush trigger | Policy executor detects cloud_reachable = true → auto-flush to cloud.ingest |
| Integrity test | test_buffer_flush.py — 200 events buffered during outage; 200 delivered on recovery |
| Unit test | test_nats_buffer.py — push/pop, overflow policy, zero critical loss at 700 events into 500 buffer |
| Pass criteria | Zero critical events lost at any buffer fill level; 100% flush integrity on recovery |

## 4.6  Alert Router

The alert router implements a four-layer alerting hierarchy. Each layer operates independently — failure of one layer does not prevent others from firing. All alerts are logged to S3 regardless of delivery status.

| Layer | Channel | Technology | Condition | Requires Internet |
| --- | --- | --- | --- | --- |
| 1 — Physical | GPIO LED / relay | Jetson.GPIO (future hardware) | Any ANOMALY during EDGE_ONLY | No — purely electrical |
| 2 — LAN | MQTT broadcast | paho-mqtt → Mosquitto broker | Any ANOMALY during EDGE_ONLY | No — LAN only |
| 3 — LAN | Web dashboard | FastAPI + HTML auto-refresh | Any ANOMALY during EDGE_ONLY | No — LAN only (port 8080) |
| 4 — Cloud | Lambda → SES email | boto3 Lambda invoke → AWS SES | CRITICAL anomaly only | Yes — falls back to MQTT if unavailable |

## 4.7  Kafka Producer and Topics

| Property | Specification |
| --- | --- |
| Technology | confluent-kafka-python 2.x — synchronous producer with error handling |
| Bridge pattern | Subscribes to NATS cloud.ingest → produces to Kafka topic sensors.events |
| Serialisation | JSON serialisation — schema validated before produce |
| Error handling | Failed produces retry 3 times with exponential backoff then log to S3 dead-letter |
| Throughput target | 1,000 events/second sustained — benchmarked in integration test |
| Topics created | sensors.events (all events), sensors.anomalies (anomaly-only), sensors.explain (SHAP requests) |
| Unit test | test_kafka_producer.py — serialisation, topic routing, error handling |
| Pass criteria | Zero message loss at 1,000 events/s; P99 produce latency < 100ms |

## 4.8  Spark Streaming Processor

| Property | Specification |
| --- | --- |
| Technology | Apache Spark 3.5 Structured Streaming — readStream from Kafka |
| Trigger interval | processingTime = 5 minutes — batches writes to control S3 PUT request count |
| Transformations | Parse JSON → validate schema → enrich with partition_date → compute anomaly_rate window |
| Output | Delta Lake write to S3: s3a://bucket/delta-lake/events/ partitioned by date+mode |
| Checkpoint | s3a://bucket/checkpoints/ — enables restart recovery without data loss |
| SHAP routing | Anomaly events additionally published to sensors.explain topic for SHAP processing |
| Spark UI | Available at localhost:8081 during evaluation — screenshot for thesis |
| Unit test | test_spark_processor.py — stream parsing, window aggregation, Delta write |
| Pass criteria | Events appear in Delta Lake within 5 seconds; zero data corruption; schema consistent |

## 4.9  SHAP Explainer

| Property | Specification |
| --- | --- |
| Technology | SHAP 0.44 TreeExplainer — NOT KernelExplainer (too slow) |
| API | FastAPI endpoint POST /explain — accepts anomaly event + feature vector |
| Latency target | P95 < 500ms — benchmarked in Sprint 7 before use in Hybrid mode |
| Background samples | Pre-computed during training — not computed at runtime |
| Output | {anomaly_id, shap_values[], feature_names[], explanation_ms} |
| Hybrid mode use | Only invoked for ANOMALY events when in HYBRID mode — not for NORMAL events |
| Fallback | If P95 > 500ms: switch to raw feature importance scores (model.feature_importances_) |
| Unit test | test_shap_explainer.py — output shape, feature names, score range, latency |
| Pass criteria | P95 explanation latency < 500ms; all features explained; values sum to ~1.0 |

## 4.10  MLflow Model Registry

| Property | Specification |
| --- | --- |
| Technology | MLflow 2.x — local tracking server at localhost:5000 |
| Artifact storage | s3://bucket/mlflow-artifacts/ — model files persist to AWS S3 |
| Logged per run | F1 score, precision, recall, n_estimators, contamination, training timestamp, dataset |
| Model registration | Best F1 run registered as 'IsolationForest-Edge' model — versioned |
| Model loading | Policy executor loads model by version string at startup |
| Retraining trigger | Lambda drift_detector flags high anomaly rate → manual retraining job |
| Unit test | test_mlflow_registry.py — experiment logging, model registration, version retrieval |
| Pass criteria | Model loadable by version string; all metrics match training output |

# 5.  AWS Integration — Option B Design

AdaptiveOrchestrator uses AWS Free Tier for three specific services only. All compute-intensive components (Kafka, Spark, Inference Engine, Policy Executor) run locally in Docker to avoid MSK and EMR costs. AWS is used exclusively for storage, monitoring, and event-driven serverless functions.

AWS Cost Commitment: S3 (~£0–£2/month at evaluation scale), CloudWatch (free — 10 metrics, 1M API calls), Lambda (free — 1M invocations/month). MSK and EMR are NOT used. Total expected AWS spend across the 8-week project: under £5.

## 5.1  S3 Bucket Structure

S3 Bucket Layout s3://adaptive-orchestrator-ajith-2026/ ├── delta-lake/ │ ├── events/ ← Spark Delta Lake writes (partitioned by date+mode) │ └── shap-results/ ← SHAP explanation outputs ├── mlflow-artifacts/ │ └── IsolationForest-Edge/ ← Versioned model files from MLflow ├── evaluation-logs/ │ ├── s1_results.json ← Scenario 1: Normal baseline │ ├── s2_results.json ← Scenario 2: Blackout (30min outage) │ ├── s3_results.json ← Scenario 3: Satellite link │ ├── s4_results.json ← Scenario 4: CPU storm │ └── s5_results.json ← Scenario 5: Cost audit ├── drift-reports/ │ └── report_YYYYMMDD.json ← Daily Lambda drift detection reports └── models/ └── isolation_forest.pkl ← Edge model binary backup

## 5.2  CloudWatch Metrics Published

| Metric Name | Namespace | Unit | Published By | Frequency | Free Tier Impact |
| --- | --- | --- | --- | --- | --- |
| OperationalMode | AdaptiveOrchestrator | None (1/2/3) | Policy Executor | Every 60s | 1 of 10 metrics |
| NetworkLatencyMs | AdaptiveOrchestrator | Milliseconds | Policy Executor | Every 60s | 1 of 10 metrics |
| EdgeCPUPercent | AdaptiveOrchestrator | Percent | Policy Executor | Every 60s | 1 of 10 metrics |
| AnomalyRate | AdaptiveOrchestrator | Percent | Policy Executor | Every 60s | 1 of 10 metrics |
| BufferSize | AdaptiveOrchestrator | Count | Policy Executor | Every 60s | 1 of 10 metrics |
| ModeTransition | AdaptiveOrchestrator | Count | Policy Executor | On transition | Event counter |

Free tier allows 10 custom metrics and 1,000,000 API calls per month. At 1 publish per minute with 5 metrics = 7,200 API calls per month = 0.72% of allowance. ModeTransition events add approximately 50 calls per evaluation day. Comfortably within free tier.

## 5.3  Lambda Functions

| Function | Trigger | Runtime | Memory | Timeout | IAM Permissions | Free Tier Impact |
| --- | --- | --- | --- | --- | --- | --- |
| anomaly_alert_dispatcher | Direct boto3 invoke from Policy Executor during EDGE_ONLY + ANOMALY | Python 3.11 | 128MB | 30s | SES send, S3 write, CloudWatch logs | ~100 invocations/month |
| drift_detector | CloudWatch EventBridge — daily at 02:00 UTC | Python 3.11 | 128MB | 60s | CloudWatch read, S3 read/write | 30 invocations/month |

# 6.  Network Simulation Design

All network conditions are simulated using tc netem (Traffic Control Network Emulator) applied to the Docker bridge network interface. This is a Linux kernel subsystem — no external hardware required. Conditions are injected at the kernel level, meaning the application software cannot distinguish simulated from real network conditions.

| Scenario | Network Profile | tc netem Command | Orchestrator Expected Response | Thesis Metric |
| --- | --- | --- | --- | --- |
| S1: Normal Baseline | No degradation — clean network | sudo tc qdisc del dev $IF root | CLOUD_OPTIMISED — full pipeline active | F1 > 0.95; baseline latency < 20ms |
| S2: The Blackout | 100% packet loss × 30 minutes | sudo tc qdisc add dev $IF root netem loss 100% | EDGE_ONLY within 5s; buffer fills; Lambda alert fired | Edge autonomy > 99%; zero detection gaps |
| S3: Satellite Link | 900ms ±150ms delay, 1.5% loss × 10min | netem delay 900ms 150ms distribution normal loss 1.5% | EDGE_ONLY triggered; recovery to CLOUD when restored | Mode switch latency < 5s; F1 > 0.85 during degradation |
| S4: CPU Storm | 90% CPU load × 10min (stress-ng) | stress-ng --cpu 7 (7 of 8 logical cores) | EDGE_ONLY from CPU threshold; load shed activated | Load shed triggers at 80%; inference latency < 100ms |
| S5: Cost Audit | Varying anomaly rate 10%→50% | Dataset injection rate varied; no network degradation | HYBRID mode for critical anomalies; CLOUD for normal | Cloud egress events vs static baseline — target 40% reduction |

# 7.  Dashboard and Monitoring Design

Four tools serve different monitoring purposes. Each is used at a specific project phase for a specific audience. No single tool replaces the others.

| Tool | Purpose | Audience | When Built | Works Offline | Thesis Use |
| --- | --- | --- | --- | --- | --- |
| Grafana + Prometheus | Live edge monitoring during evaluation scenarios | Developer during evaluation | Week 5 | ✅ Yes — fully local | Screenshot during Scenario 2/3 showing mode transitions |
| AWS CloudWatch | Cloud metrics over time — 5 custom metrics | Dr. Thushari, examiner | Week 5 | ❌ Requires internet | Screenshot showing AWS console with real metrics over time |
| Streamlit | Interactive evaluation results presentation | Defence audience | Week 8 | ✅ Yes — local Python | Live demo during defence — all 5 scenario results interactive |
| Matplotlib | Static publication-quality thesis figures | Thesis document reader | Week 8 | ✅ Yes | PNG figures embedded in thesis Word document |
| FastAPI Dashboard | Simple local alert display during LAN outage | Operator on LAN | Week 4 | ✅ Yes — LAN only | Supplementary screenshot showing alert visibility during outage |

## 7.1  Prometheus Metrics Exposed by Each Component

| Component | Metric Name | Type | Description |
| --- | --- | --- | --- |
| Policy Executor | orchestrator_mode | Gauge | Current mode: 1=EDGE_ONLY, 2=HYBRID, 3=CLOUD_OPTIMISED |
| Policy Executor | mode_transitions_total | Counter | Cumulative mode transitions since startup |
| Policy Executor | routing_decisions_total | Counter | Total inference results routed |
| Context Monitor | network_latency_ms | Gauge | Current measured latency to cloud_node |
| Context Monitor | edge_cpu_percent | Gauge | Current CPU utilisation |
| Context Monitor | edge_ram_percent | Gauge | Current RAM utilisation |
| Context Monitor | cloud_reachable | Gauge | 1=reachable, 0=unreachable |
| Inference Engine | inference_latency_ms | Histogram | Per-inference latency distribution |
| Inference Engine | anomaly_score | Histogram | Distribution of anomaly scores |
| Inference Engine | anomalies_detected_total | Counter | Cumulative anomaly count |
| NATS Buffer | buffer_size_current | Gauge | Current number of events in buffer |
| NATS Buffer | buffer_events_dropped_total | Counter | Events dropped due to overflow |
| NATS Buffer | buffer_flush_events_total | Counter | Events delivered on network recovery |

# 8.  Alerting Architecture

The alert router implements a four-layer hierarchy designed so that every layer operates independently. A WAN outage that prevents Lambda invocation does not prevent MQTT broadcast. The system degrades gracefully — each layer that fails falls through to the next available layer.

ANOMALY detected during EDGE_ONLY mode │ ▼ ┌─────────────────────────────────────────────────┐ │ Alert Router — Severity Check │ │ score > 0.85 → CRITICAL │ │ score > 0.60 → HIGH │ └─────────────────────────────────────────────────┘ │ ┌──────────┼──────────┬──────────────┐ ▼ ▼ ▼ ▼ Layer 1 Layer 2 Layer 3 Layer 4 GPIO LED MQTT FastAPI Lambda (future) broadcast Dashboard → SES Email Mosquitto port 8080 Works: Works: Works: Works: always LAN up LAN up Internet up no WAN no WAN Falls back to needed needed MQTT if down │ ▼ ALL layers → S3 alert log (async, best-effort)

# 9.  Configuration Reference

All configurable values live in config/settings.py. No threshold, URL, or credential is hardcoded anywhere in the codebase. Environment variables override config values for sensitive items (AWS credentials, phone numbers).

| Parameter | Default Value | Evidence Basis | Where Used |
| --- | --- | --- | --- |
| LATENCY_THRESHOLD_MS | 500 | Industrial real-time control standards; validated by threshold sweep experiment (Week 2) | Context Monitor → Policy Executor mode decision |
| CPU_THRESHOLD_PCT | 80 | Embedded systems thermal management literature; validated by CPU sweep experiment (Week 2) | Context Monitor → Policy Executor load-shed decision |
| ANOMALY_SCORE_THRESHOLD | 0.60 | Normalised IF score baseline; tuned during CWRU training in Week 1 | Inference Engine decision boundary |
| CRITICAL_SCORE_THRESHOLD | 0.85 | Threshold above which HYBRID mode is triggered and Lambda alert fired | Policy Executor Hybrid routing; Alert Router severity |
| BUFFER_MAX_SIZE | 5000 | 50 seconds at 100Hz — covers expected outage detection + mode switch time | NATS Buffer capacity limit |
| BUFFER_CRITICAL_RESERVE | 500 | 10% of buffer always reserved for ANOMALY events | NATS Buffer overflow policy |
| REPLAY_RATE_HZ | 100 | Standard industrial vibration sensor sampling rate | Sensor Simulator replay speed |
| CW_PUBLISH_INTERVAL_S | 60 | Minimises CloudWatch API calls while maintaining 1-minute resolution | CloudWatch Publisher loop interval |
| SPARK_TRIGGER_INTERVAL | 5 minutes | Controls S3 PUT request count — keeps within free tier 2k PUT limit | Spark Streaming write trigger |
| AWS_REGION | ap-south-1 | Closest AWS region to Sri Lanka (Mumbai) | All boto3 clients |

# 10.  Docker Compose Service Reference

| Service | Container Name | Image | CPU Limit | RAM Limit | Ports | Networks |
| --- | --- | --- | --- | --- | --- | --- |
| nats | nats_broker | nats:latest | 0.5 | 512MB | 4222, 8222 | edge_cloud_net |
| edge_node | edge_node | python:3.10-slim | 1.0 | 4GB | 8000, 8080 | edge_cloud_net |
| cloud_node | cloud_node | python:3.10-slim | 2.0 | 6GB | 8001, 8002 | edge_cloud_net |
| mosquitto | mqtt_broker | eclipse-mosquitto | 0.25 | 256MB | 1883 | edge_cloud_net |
| prometheus | prometheus | prom/prometheus:latest | 0.5 | 512MB | 9090 | edge_cloud_net |
| grafana | grafana | grafana/grafana:latest | 0.5 | 512MB | 3000 | edge_cloud_net |
| zookeeper | zookeeper | confluentinc/cp-zookeeper:7.5 | 0.5 | 1GB | 2181 | edge_cloud_net |
| kafka | kafka | confluentinc/cp-kafka:7.5 | 1.0 | 2GB | 9092 | edge_cloud_net |
| spark_master | spark_master | bitnami/spark:3.5 | 1.0 | 2GB | 8081 | edge_cloud_net |
| spark_worker | spark_worker | bitnami/spark:3.5 | 2.0 | 2GB | — | edge_cloud_net |
| mlflow | mlflow_server | ghcr.io/mlflow/mlflow | 0.5 | 1GB | 5000 | edge_cloud_net |

Total resource allocation: 10.0 vCPU limit across all containers (your machine has 8 logical processors — limits are ceiling values, not guaranteed allocations), 20.5GB RAM limit. Your WSL2 has 7.6GB available — Spark worker RAM limit will be constrained to 1.5GB in practice. This is acceptable for evaluation-scale workloads.

# 11.  Risk Register

| # | Risk | Severity | Mitigation | Resolution |
| --- | --- | --- | --- | --- |
| R1 | Edge compute overload | Critical | Docker resource limits simulate Jetson Nano constraints | ✅ Resolved — Sprint 0 passed. CPU 0.0% at idle. |
| R2 | Model drift during long outages | High | Scope 99% autonomy claim to outages <60 minutes in thesis | ✅ Thesis language — no code change needed |
| R3 | AWS cost spiral | High | MSK and EMR not used. S3+CloudWatch+Lambda only. Billing alert at $10. | ✅ Architectural decision — zero risk |
| R4 | SHAP latency too slow | High | Benchmark TreeExplainer in Sprint 7. Fallback to feature_importances_. | ⏳ Week 6 — benchmark before Hybrid mode evaluation |
| R5 | Fixed thresholds arbitrary | High | Threshold sweep experiment produces evidence-based values | ⏳ Week 2 — sweep validates 500ms and 80% |
| R6 | Dataset scope too narrow | Medium | Acknowledge CWRU+SKAB as benchmarks. Synthetic multi-sensor expansion. | ✅ Thesis scoping language written |
| R7 | NATS buffer overflow | Medium | Drop-NORMAL policy. Critical reserve. Overflow unit test. | ⏳ Week 4 — buffer overflow test |
| R8 | Docker overhead distorts latency | Medium | Disclose in thesis methodology. Measure with/without --network host. | ✅ Thesis language written |
| R9 | tc netem burst loss unrealistic | Low | Use Markov state loss model. Acknowledge limitation in methodology. | ⏳ Week 1 — use netem loss state model |
| R10 | MLOps loop out of M.Sc. scope | Low | Manual retraining trigger. Automated drift as future work. | ✅ Scoping decision made |

# 12.  Thesis Chapter Mapping

Every component and every evaluation scenario maps directly to a specific thesis chapter. This table ensures that when writing begins in Week 8, the evidence for every claim already exists as a logged artefact.

| Thesis Chapter | Content Source | Evidence Artefact | Figure / Table |
| --- | --- | --- | --- |
| Ch1: Introduction | Research context, problem statement, three failure modes | Literature review — 9 papers | Table: Static vs dynamic comparison |
| Ch2: Literature Review | 9 IEEE papers — static orchestration evidence | Literature review document | Table: Gap analysis per paper |
| Ch3: System Design | This architecture document | Architecture document v1.0 | Figure: 4-layer architecture diagram |
| Ch3: System Design | Policy decision logic | Policy executor code + flowchart | Figure: Decision flowchart |
| Ch4: Implementation | All 11 component implementations | GitHub repository | Code listings: Key components |
| Ch4: Implementation | Docker Compose configuration | docker-compose.yml | Table: Service reference |
| Ch5: Evaluation Setup | tc netem scenario scripts | scripts/scenarios/*.sh | Table: 5 evaluation scenarios |
| Ch5: Evaluation Setup | Threshold sweep results | logs/threshold_sweep.json | Figure: F1 vs latency curve |
| Ch6: Results | S1 baseline scenario | logs/s1_results.json | Table: Baseline metrics |
| Ch6: Results | S2 blackout scenario | logs/s2_results.json | Figure: Edge autonomy during 30min outage |
| Ch6: Results | S3 satellite link scenario | logs/s3_results.json | Figure: Mode transition timeline |
| Ch6: Results | S4 CPU storm scenario | logs/s4_results.json | Figure: Load-shed trigger latency |
| Ch6: Results | S5 cost audit scenario | logs/s5_results.json | Figure: Cost reduction bar chart |
| Ch6: Results | CloudWatch dashboard | AWS Console screenshot | Figure: Live cloud metrics during evaluation |
| Ch6: Results | Grafana dashboard | Browser screenshot | Figure: Real-time mode transitions |
| Ch7: Discussion | All results vs targets | parse_evaluation_results.py output | Table: Targets vs achieved |
| Ch7: Discussion | Risks 2,6,8,10 — limitations | This document Section 11 | Paragraph: Limitations and future work |
| Ch8: Conclusion | Research contribution summary | Policy executor + evaluation | Paragraph: Gap addressed |

# 13.  Document Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 0.1 | February 2026 | Ajith Kumara | Initial architecture proposal — presented to Dr. Thushari Silva |
| 0.2 | February 2026 | Ajith Kumara | Feasibility concerns addressed — sensors, satellite latency, outage simulation |
| 0.3 | February 2026 | Ajith Kumara | Hardware decision — laptop simulation with Docker resource constraints |
| 0.4 | February 2026 | Ajith Kumara | AWS Option B selected — S3 + CloudWatch + Lambda only |
| 0.5 | February 2026 | Ajith Kumara | Dashboard stack finalised — Grafana + CloudWatch + Streamlit + Matplotlib |
| 1.0 | February 2026 | Ajith Kumara | Final architecture document — approved for implementation |

This document is the single source of truth for the AdaptiveOrchestrator architecture.

Any design decision not recorded here has not been made.

AdaptiveOrchestrator  |  Ajith Kumara  |  University of Moratuwa  |  2026